# Swissphone Classic Archive - Current Research Summary

Version: v0.1
Status: Research in Progress

## Current Findings

Device: Swissphone Memo (4 m BOS version, presumed).

IF IC identified: Motorola MC3367DW.

Quartz marking: QS421 / 20.94 / TEW5E.

Measured oscillator peaks: 20.99 MHz, 41.88 MHz, 83.763 MHz, 83.781 MHz (-92 dBm).

Oscillator only active when both PCB assemblies are connected.

Programming interface idle voltage: Pin2 ≈3.0 V, Pin3 ≈3.0 V.

Resistance to GND (off): Pin2 ≈2.3 MΩ, Pin3 ≈184 kΩ.

Diode test: Pin→GND OL (~2.9 V), GND→Pin ≈0.24 V.

Hypothesis: 3 V UART-level interface (not RS-232).

Logic Analyzer TEST001: CH0=Pin2, CH1=Pin3. No valid UART detected during power-on. Short transitions observed on Pin2 during startup.

## Next planned work

Analyze TEST002-TEST004 captures.

Search and test PSW329.

Determine baud rate and protocol.

Investigate Pin1 and Pin4.

Design measurement/programming cradle.